EXP NO – 15

TRANSACTION LEVEL MODELING (TLM) USING SYSTEMC

AIM 

To model and simulate Transaction Level Modeling (TLM) using SystemC, demonstrating communication between an initiator and a target through high-level transactions instead of signal-level communication.

PROGRAM

#include <systemc>

#include <tlm>

#include <tlm_utils/simple_initiator_socket.h>

#include <tlm_utils/simple_target_socket.h>

using namespace sc_core;

using namespace tlm;

using namespace std;

struct Target : sc_module

{

tlm_utils::simple_target_socket<Target> socket;

SC_CTOR(Target) : socket("socket")

{

socket.register_b_transport(this, &Target::b_transport);

}

void b_transport(tlm_generic_payload &trans, sc_time &delay)

{

int *data;

data = (int*)trans.get_data_ptr();

cout << "Target Received Data = "

<< *data

<< " at "

<< sc_time_stamp()

<< endl;

delay += sc_time(10, SC_NS);

}

};

struct Initiator : sc_module

{

tlm_utils::simple_initiator_socket<Initiator> socket;

SC_CTOR(Initiator)

{

SC_THREAD(process);

}

void process()

{

tlm_generic_payload trans;

int data = 100;

trans.set_command(TLM_WRITE_COMMAND);

trans.set_address(0);

trans.set_data_ptr((unsigned char*)&data);

trans.set_data_length(sizeof(data));

sc_time delay = SC_ZERO_TIME;

cout << "Initiator Sending Data = "

<< data

<< endl;

socket->b_transport(trans, delay);

wait(delay);

cout << "Transaction Completed at "

<< sc_time_stamp()

<< endl;

}

};

SC_MODULE(Top)

{

Initiator initiator;

Target target;

SC_CTOR(Top)

: initiator("Initiator"),

target("Target")

{

initiator.socket.bind(target.socket);

}

};

int sc_main(int argc, char* argv[])

{

Top top("Top");

sc_start();

return 0;

}

SIMULATION

OUTPUT :